VOLUME 4.2

FINANCIAL OFFER TEMPLATES

LUMP SUM CONTRACTS

VOLUME 4.2

FINANCIAL OFFER TEMPLATES

LUMP SUM CONTRACTS

Introduction

1.	The breakdown of the lump-sum price (Volume 4.2.3) is the itemised list of prices showing the build-up of the price in a lump-sum contract. This breakdown of the lump-sum price does not derogate in any way to the clause stating that, in a lump-sum contract, the total contract price remains fixed irrespective of the quantity of work actually carried out.

The amounts due will be calculated:

The breakdown must coincide with the payment-definition chosen in Article 49 of the special conditions:

2.	The item description given in the breakdown of the lump-sum price in no way limits the contractor’s obligations under the contract to provide all the works described elsewhere.

3.	The prices of the breakdown of the lump-sum price include all incidental and contingent expenses and all risks necessary to construct, complete and maintain all works in accordance with the contract. Unless separate items are provided in the breakdown of the lump-sum price, prices include all costs involved in the various items of the breakdown.

4.	The lump–sum price and the prices of the breakdown of the lump-sum price are all-inclusive and include any non-exonerated tax or fiscal duty.

VOLUME 4.2.2 — SUMMARY

| Description | Amount EUR |
| --- | --- |
| Total of lump-sum price without VAT |  |
| VAT amount |  |
| TOTAL PRICE included VAT |  |

VOLUME 4.2.3 — BREAKDOWN OF THE LUMP-SUM PRICE

| PRE-MEASURE BILL FOR REGULATION OF THE BREGALNICA RIVER BED |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Arrangement of the bed of the river Bregalnica from km 0+000.00 to km 0+300.00 |  |  |  |  |  |
| No | Description of the works | Single measure | Quantity | Unit price | EUR in total |
| 1 | PREPARATORY WORKS |  |  |  |  |
| 1.1 | Marking and staking out the projected route of the river bed by connecting the existing trigonometric network | m | 300.00 |  |  |
|  | TOTAL 1 |  |  |  |  |
| 2 | PREPARATORY WORK |  |  |  |  |
| 2.1 | Complete clearing of the riverbed and the coastal belt from bushy vegetation | m2 | 240.00 |  |  |
| 2.2 | Machine cutting of trees, pruning of branches and side stacking |  |  |  |  |
|  | branches Ф 10-20см | piece | 120.00 |  |  |
|  | branches Ф 20-30см | piece | 24.00 |  |  |
|  | TOTAL 2 |  |  |  |  |
| 3 | EARTH WORKS |  |  |  |  |
| 3.1 | Mechanical excavation of category III land in a wide excavation in water and partly in dry land for the formation of the regulated profile by planning and by transferring the excavated material to the protective embankment at a maximum distance of up to 50m (according to the table for soil masses)V=7167.67m3/s, од км 0+000.00 до км 0+300.00 |  |  |  |  |
| 3.1.1 | Mechanical excavation 90% | m3 | 6450.90 |  |  |
| 3.1.2 | Manual digging of the ground 10% to shape the bed | m3 | 716.77 |  |  |
| 3.2 | Manual shaping, planning of the base of the slopes and the bottom according to the required geometry | m2 | 17100.00 |  |  |
| 3.3. | Formation and planning of the platforms of the major bed with material from the excavation in layers of 30 cm with compaction to the required compaction, with a minimum compressibility module Mvmin=50MPa | m3 | 1436.09 |  |  |
| 3.4 | Embankment of excavated material for major in layers of 30 cm with compaction to the required compaction, with a minimum modulus of compressibility Mvmin=50MPa | m3 | 161.04 |  |  |
| 3.5 | Forming and planning of the embankments of the minor bed with roughly arranged stone d=30-50cm | m3 | 1669.53 |  |  |
|  | TOTAL 3 |  |  |  |  |
| 4 | SECURING THE TRANSVERSE PROFILE |  |  |  |  |
| No | Description of the works | single measure | quantity |  |  |
| 4.1 | Category III mechanical excavation of earth for the execution of shearing and lining of the slopes by transferring the excavated material to the protective embankments of the bed | m3 | 1617.54 |  |  |
| 4.2 | Category III mechanical earth excavation for the performance of lining of the major trough | m3 | 414.82 |  |  |
| 4.3 | Category III mechanical earth excavation for the construction of quay walls | m3 | 516.63 |  |  |
| 4.4 | Making a base from lean concrete d=15cm | m3 | 141.59 |  |  |
| 4.5 | Production of cladding on the slopes with procurement, transport and placement of regular stone d=30cm with joints filled with cement mortar 1:3 (minor) | m3 | 419.11 |  |  |
| 4.6 | Procurement, transport and installation of stone for lining the major trough (coarse dry stone d=30 cm) | m3 | 318.20 |  |  |
| 4.7 | Making the shears of the coating and the clearer part of the coating of the slope with procurement, transport and installation of coarsely graded dry crushed stone d=30-50cm | m3 | 1380.00 |  |  |
| 4.8 | Loading and removal of excess excavated material to a landfill up to 10 km away (the location of the landfill is determined by the municipality) | m3 | 7779.15 |  |  |
| 4.9 | Procurement, transportation and construction of reinforced concrete quay walls along the left bank with a total length of 48.85+32=80.85m with hydrotechnical concrete MB30 | m3 | 176.25 |  |  |
| 5.0 | Procurement, transport and installation of structural reinforcement RA 400/500 for the quay walls | kg | 6430.00 |  |  |
| 5.1 | Compaction of working trenches for quay walls with excavated material in layers of 30 cm with compaction to the required compaction, with a minimum compression modulus Mvmin=50MPa | m3 | 340.38 |  |  |
|  | TOTAL 4 |  |  |  |  |
| 5 | BAND THRESHOLD KM 0+057.00 |  |  |  |  |
| No | Description of the works | Single measure | Quantity |  |  |
| 5.1 | EARTH WORKS |  |  |  |  |
| 5.1.1 | Mechanical excavation of land III category in a narrow scope in water with a depth of more than 1.0m | m3 | 65.69 |  |  |
| 5.1.2 | Loading and taking the excavated material to a landfill up to 10 km away (the location of the landfill is determined by the municipality) | m3 | 65.69 |  |  |
|  | TOTAL 5.1 |  |  |  |  |
| 5.2 | INSTALLATION WORKS |  |  |  |  |
| 5.2.1 | Procurement, transportation and installation of gabions for making thresholds for stabilization of the riverbed with dimensions: |  |  |  |  |
|  | 1.0/2.0/1.0m | пар. | 23 |  |  |
|  | 2.0/0.5/1.0m | пар. | 16 |  |  |
|  | 0.5/0.5/1.0m | пар. | 1 |  |  |
| 5.2.2 | Procurement, transportation and embedding of stone in cement mortar to form the projected profile | m3 | 5.45 |  |  |
| 5.2.3 | Incorporation of roughly arranged stone in a length of 3.0m downstream of the threshold d=30cm | m3 | 13.77 |  |  |
|  | TOTAL 5.2 |  |  |  |  |
|  | TOTAL 5 |  |  |  |  |
| 6 | BAND THRESHOLD КМ 0+110.00 |  |  |  |  |
| No | Description of the works | Single measure | quantity |  |  |
| 6.1 | EARTH WORKS |  |  |  |  |
| 6.1.1 | Mechanical excavation of land III category in a narrow scope in water with a depth of more than 1.0m | m3 | 54.22 |  |  |
| 6.1.2 | Loading and taking the excavated material to a landfill up to 10 km away (the location of the landfill is determined by the municipality) | m3 | 54.22 |  |  |
|  | TOTAL6.1 |  |  |  |  |
| 6.2 | INSTALLATION WORKS |  |  |  |  |
| 6.2.1 | Procurement, transportation and installation of gabions for making thresholds for stabilization of the river bed with dimensions: |  |  |  |  |
|  | 1.0/2.0/1.0m | пар. | 23 |  |  |
|  | 2.0/0.5/1.0m | пар. | 16 |  |  |
|  | 1.0/1.0/1.0m | пар. | 1 |  |  |
|  | 0.5/1.0/1.0m | пар. | 3 |  |  |
| 6.2.2 | Procurement, transportation and embedding of stone in cement mortar to form the projected profile | m3 | 6.50 |  |  |
| 6.2.3 | Incorporation of roughly arranged stone in a length of 3.0m downstream of the threshold d=30cm | m3 | 13.77 |  |  |
|  | TOTAL 6.2 |  |  |  |  |
|  | TOTAL 6 |  |  |  |  |
| 7 | BAND THRESHOLD КМ 0+170.00 |  |  |  |  |
| No | Description of the works | Single measure | quantity |  |  |
| 7.1 | EARTH WORKS |  |  |  |  |
| 7.1.1 | Mechanical excavation of land III category in a narrow scope in water with a depth of more than 1.0m | m3 | 59.57 |  |  |
| 7.1.2 | Loading and taking the excavated material to a landfill up to 10 km away (the location of the landfill is determined by the municipality) | m3 | 59.57 |  |  |
|  | TOTAL 7.1 |  |  |  |  |
| 7.2 | INSTALLATION WORKS |  |  |  |  |
| 7.2.1 | Procurement, transportation and installation of gabions for making thresholds for stabilization of the river bed with dimensions: |  |  |  |  |
|  | 1.0/1.0/2.0m | piece | 24.00 |  |  |
|  | 2.0/0.5/1.0m | piece | 18.00 |  |  |
|  | 1.0/0.5/1.0m | piece | 1.00 |  |  |
| 7.2.2 | Procurement, transportation and embedding of stone in cement mortar to form the projected profile | m3 | 6.64 |  |  |
| 7.2.3 | Incorporation of roughly arranged stone in a length of 3.0m downstream of the threshold d=30cm | m3 | 13.77 |  |  |
|  | TOTAL 7.2 |  |  |  |  |
|  | TOTAL 7 |  |  |  |  |
| 8 | BAND THRESHOLD КМ 0+230.00 |  |  |  |  |
| No | Description of the works | Single measure | quantity |  |  |
| 8.1 | EARTH WORKS |  |  |  |  |
| 8.1.1 | Mechanical excavation of land III category in a narrow scope in water with a depth of more than 1.0m | m3 | 59.11 |  |  |
| 8.1.2 | Loading and taking the excavated material to a landfill up to 10 km away (the location of the landfill is determined by the municipality) | m3 | 59.11 |  |  |
|  | TOTAL 8.1 |  |  |  |  |
| 8.2 | INSTALLATION WORKS |  |  |  |  |
| 8.2.1 | Procurement, transportation and installation of gabions for making thresholds for stabilization of the river bed with dimensions: |  |  |  |  |
|  | 1.0/1.0/2.0m | piece | 22.00 |  |  |
|  | 2.0/0.5/1.0m | piece | 18.00 |  |  |
|  | 1.0/1.0/1.0m | piece | 2.00 |  |  |
| 8.2.2 | Procurement, transportation and embedding of stone in cement mortar to form the projected profile | m3 | 6.41 |  |  |
| 8.2.3 | Incorporation of roughly arranged stone in a length of 3.0m downstream of the threshold d=30cm | m3 | 13.77 |  |  |
|  | TOTAL 8.2 |  |  |  |  |
|  | TOTAL 8 |  |  |  |  |
| 9 | BAND THRESHOLD КМ 0+290.00 |  |  |  |  |
| No | Description of the works | Single measure | quantity |  |  |
| 9.1 | EARTH WORKS |  |  |  |  |
| 9.1.1 | Mechanical excavation of land III category in a narrow scope in water with a depth of more than 1.0m | m3 | 62.51 |  |  |
| 9.1.2 | Loading and taking the excavated material to a landfill up to 10 km away (the location of the landfill is determined by the municipality) | m3 | 62.51 |  |  |
|  | TOTAL 9.1 |  |  |  |  |
| 9.2 | INSTALLATION WORKS |  |  |  |  |
| 9.2.1 | Procurement, transportation and installation of gabions for making thresholds for stabilization of the river bed with dimensions: |  |  |  |  |
|  | 1.0/1.0/2.0m | piece | 25.00 |  |  |
|  | 2.0/0.5/1.0m | piece | 18.00 |  |  |
|  | 1.0/0.50/1.0m | piece | 1.00 |  |  |
| 9.2.2 | Procurement, transportation and embedding of stone in cement mortar to form the projected profile | m3 | 6.19 |  |  |
| 9.2.3 | Installation of rough stone in a length of 3.0m downstream of the threshold d=30-50cm | m3 | 13.77 |  |  |
|  | TOTAL 9.2 |  |  |  |  |
|  | TOTAL 9 |  |  |  |  |
| 16 | FLOOD INFLOW INTO BREGALNICA RIVER КМ 0+034.18 |  |  |  |  |
| No | Description of the works | Single measure | quantity |  |  |
| 16.1 | EARTH WORKS |  |  |  |  |
| 16.1.1 | Mechanical excavation of category III soil in a narrow scope with a depth of 1.0m for the cascade thresholds and the torrent to the mouth of the Bregalnica river | m3 | 170.83 |  |  |
| 16.1.2 | Loading and taking the excavated material to a landfill up to 10 km away (the location of the landfill is determined by the municipality) | m3 | 170.83 |  |  |
|  | TOTAL 16.1 |  |  |  |  |
| 16.2 | MASONRY WORKS |  |  |  |  |
| 16.2.1 | Procurement of transportation and installation of stone in cement mortar for the construction of the torrent |  |  |  |  |
|  | bottom | m3 | 12.64 |  |  |
|  | walls | m3 | 25.39 |  |  |
| 16.3.2 | Procurement, transportation and installation of lean concrete d=0.20m, as a base for laying the stone lining at the bottom | m3 | 8.43 |  |  |
| 16.2.2 | Procurement, transportation and installation of gabions for the walls from the outlet of the culvert under the road to the cascade threshold |  |  |  |  |
|  | 1.0/1.0/0.50m | piece | 2 |  |  |
|  | 1.0/0.5/0.5m | piece | 2 |  |  |
| 16.2.3 | Procurement, transport and installation of lean concrete for placing the gabions | m3 | 0.21 |  |  |
|  | TOTAL 16.2 |  |  |  |  |
| 16.3 | CONCRETE WORKS |  |  |  |  |
| 16.3.1 | Procurement, transport and installation of MB 30 for making a cascade threshold |  |  |  |  |
|  | body of the cascade | m3 | 1.29 |  |  |
|  | foundations | m3 | 2.07 |  |  |
|  | TOTAL 16.3 |  |  |  |  |
| 16.4 | REINFORCEMENT WORKS |  |  |  |  |
|  | Procurement, transport and installation of concrete iron RA 400/500 | kg | 1198 |  |  |
|  | TOTAL 16.4 |  |  |  |  |
|  | TOTAL 16 |  |  |  |  |